海上回收着陆论文真实性、准确性与完整性审查

审查编号： SeaLandingPaper-ResearchIntegrity-Audit-v1

生成时间（UTC）： 2026-08-24T06:31:36+00:00

机器审查结论： PASS_WITH_DECLARED_LIMITATIONS

# 1. 审查目的与判定规则

本文件用于回答三个问题：正文中的结果是否由本项目代码重新计算，文献数据是否仅作为明确标注的验证目标，以及尚未闭合的模型环节是否被如实保留。审查不以‘图能画出来’为通过条件，而要求计算输入、生成代码、机器可读输出和成稿图之间可以逐项追溯。

本文结果分为四类。present_calculation 是本项目在本机重新执行得到的结果；reference_target 是从文献转录或数字化的验证目标；publishedᵢnput 是模型采用且已注明来源的文献参数；identified/proxy 是由于公开资料缺失而辨识或假设的输入。后两类不得写成原作者公布值，也不得用于声称专有模型已被完整复现。

# 2. 主要数值证据

矩形驳船 HAMS 输出包含 19 个频率、7 个浪向和 5 个甲板点。参数化网格与水静力自检全部通过。着陆中心最大垂荡 RAO 为 1.128 m/m，发生在 0.6 rad/s、90 deg。该数值由本地编译的 HAMS 输出重新计算，不取自任何参考论文。

随机波筛选包含 63 个海况-浪向组合，每个组合 100 个确定性随机相位种子，单次时长 600 s，步长 0.1 s。全矩阵最大着陆中心 P95 垂向速度为 1.505 m/s，对应 Hs3_Tp10、90 deg。谱矩归一化和显著波高线性检查来自脚本自动判定。

Chrono-Cummins 代理算例的本机计算峰值接触力为 2.678 MN，峰值缓冲行程为 46.44 mm。但最大 SMC 贯入为 0.148 m，占足垫半径的 74.1%。因此这些力学量仅用于证明开放耦合链可执行，不作为真实型号支腿载荷的验证值。

Yang 对照图中的虚线来自公开试验曲线的像素数字化，实线来自本项目模型。参数只使用 simultaneous touchdown 工况辨识，1-2-1 与 2-2 工况没有重新调参。缓冲行程峰值误差较小，但加速度峰值误差仍为 29.2%-51.9%，说明公开资料不足以重构原柔性机构。这一差异被保留，没有通过平滑、缩放或手工修改曲线消除。

# 3. 数据与文件溯源

| 文件 | 证据类别 | 用途 | 存在 | SHA-256 前缀 |
| --- | --- | --- | --- | --- |
| 新论文参考写作论文/Analysis+of+Sea-Based+Landing+Dynamics+of+Reusable+Landing+Vehicle+Considering+Mechanism+Flexibility.pdf | referenceₛource | Yang et al. validation paper | 是 | e7949d55dea5a5d9 |
| 新论文参考写作论文/Analysis+of+Sea-Based+Landing+Dynamics+of+Reusable+Landing+Vehicle+Considering+Mechanism+Flexibility.md | referenceₛource | Yang et al. text extraction | 是 | 3939e6f442f1738d |
| 海上平台火箭回收文献/065002_1_2.0002061.pdf | referenceₛource | Nargolkar and Vijayan benchmark paper | 是 | f9ba2eb6197a8ad7 |
| 海上平台火箭回收文献/065002_1_2.0002061.md | referenceₛource | Nargolkar and Vijayan text extraction | 是 | 37bc58992de0399c |
| RocketRecoveryCases/Paper_Yang_2026/reference/digitized/yang-2026-fig09-11-digitization.json | reference_target | Digitized published validation traces and raster calibration metadata | 是 | 309fb9da09933f8c |
| RocketRecoveryCases/Paper_Yang_2026/yang-2026-evidence-report.json | present_calculation | Locally generated deck-filter and source-consistency report | 是 | c00ffc3b55deb655 |
| RocketRecoveryCases/Paper_Yang_2026/identified_landing_model/yang-2026-identified-landing-report.json | present_calculation | Locally identified and predicted landing responses | 是 | da1f143fe0d4051e |
| RocketRecoveryCases/Barge_120x50/platform_config.json | present_modelᵢnput | Parameter-controlled rectangular-barge case | 是 | 50f90e7fbd6f1f1b |
| RocketRecoveryCases/Barge_120x50/validation/barge-self-check.json | present_calculation | Analytic geometry and hydrostatic checks | 是 | 06359d8905263bfc |
| RocketRecoveryCases/Barge_120x50/Output/RocketRecovery/deck-point-rao.json | present_calculation | Locally compiled HAMS deck-point RAOs | 是 | c07aad254806fce5 |
| RocketRecoveryCases/Barge_120x50/Output/RocketRecovery/sea-state-response.json | present_calculation | Spectral response integrals from HAMS RAOs | 是 | fcac3cc88010888d |
| RocketRecoveryCases/Barge_120x50/Output/RocketRecovery/wave-sensitivity.json | present_calculation | Random-phase HAMS/JONSWAP statistics | 是 | 2284f1c194ce227c |
| RocketRecoveryCases/Chrono_LeggedRecovery/chrono-stage3-two-way-report-data.json | present_calculation_withₚroxyᵢnputs | Locally executed Chrono-Cummins mechanism-proxy response | 是 | 4b983d7c6812d0ba |

完整哈希值保存在 research-integrity-audit.json。每次重算后重新生成本审查文件，可以识别输入、输出或图件是否发生变化。

# 4. 图件审查

主文图统一由 analysis/rocket_recovery/yang_2026_figures.py 生成，使用 SciencePlots 的 science 与 no-latex 样式、Times New Roman 字体，同时输出 600 dpi PNG 和可编辑 SVG。文献曲线采用灰黑虚线或点列，本文计算采用彩色实线。图件不经过 PowerPoint、Illustrator 或图像软件手工移动数据。

| 图件 | 数据性质 | PNG 像素 | SVG 字体含 Times New Roman | SHA-256 前缀 |
| --- | --- | --- | --- | --- |
| fig01-coupled-framework | Method diagram generated from the implemented data flow; no numerical result is imported. | 4299 x 2019 | 是 | ca6911012f169f15 |
| fig02-barge-geometry-mesh | Present parameterized geometry and registered deck points. | 2756 x 2259 | 是 | c714c50dadb5ec0e |
| fig03-yang-deck-filter-reproduction | Present deterministic response; dashed spectral shape is reconstructed from the published transfer function. | 4299 x 2769 | 是 | 50374283dedb022f |
| fig04-yang-identified-landing-comparison | Dashed curves are published tests digitized for validation; solid curves are present calculations. | 4235 x 3159 | 是 | d37ae985bbbd4641 |
| fig05-platform-hydrodynamic-response | Entirely present potential-flow and spectral calculations. | 4299 x 2799 | 是 | 9403cc0bef572cd6 |
| fig06-partitioned-contact-feedback | Entirely present partitioned calculation using explicitly identified published and proxy inputs. | 4299 x 2799 | 是 | 06e5fef3c36253a2 |
| fig07-random-wave-deck-statistics | Entirely present 63-case, 100-seed linear wave-screening calculation. | 4299 x 2799 | 是 | f4860076c1938eab |

第一轮图审发现并修正了两类问题：甲板滤波图曾把绝对 Welch 谱与未归一化传递函数平方直接叠加，现改为归一化谱形比较；耦合框架图曾有一条甲板运动箭头方向错误，已按 Cummins→甲板运动→Chrono、Chrono→平台接触力矩的实际接口改正。第二轮检查覆盖图例可见性、单位、峰值、坐标范围和文献/本文线型区分。

# 5. 尚未解决的真实性问题

| 严重度 | 项目 | 证据 | 对论文结论的约束 |
| --- | --- | --- | --- |
| high | Chrono contact compliance | Maximum SMC contact penetration is 0.148 m, or 74.1% of the 0.2 m footpad radius. | Stage 3A contact-force peaks are retained only as mechanism-proxy outputs and are not treated as validated full-scale leg loads. |
| high | Landing-leg geometry and absorber law | The supplied papers do not publish CAD hinge coordinates, member inertia, or a machine-readable hydropneumatic force-stroke/velocity curve. | The Chrono branch cannot be described as an Adams-equivalent reproduction of Yang or Thies. |
| medium | Yang model identifiability | The stroke observation ratio reaches its declared upper bound and acceleration peak errors remain 29.2-51.9%. | Only stroke magnitude and broad touchdown-sequence trends are supported by the public-data reduced model. |
| medium | Strong two-way closure | The accepted Stage 3A calculation uses a one-pass heave/roll/pitch correction; full 6-DOF fixed-point closure remains incomplete. | The paper reports an executable partitioned demonstration, not converged strong coupling. |
| medium | HAMS legacy benchmark portability | HywindSpar matches the registered benchmark, while Cylinder, DeepCwind, and Moonpool differ under the local GNU Fortran toolchain. | Analytic barge checks and locally reproducible outputs are valid; universal cross-toolchain equivalence is not claimed. |
| low | Random-wave interpretation | The 63-case study is a linear platform-only HAMS/JONSWAP screening calculation. | Its P95 values are not landing success probabilities or operational sea-state limits. |

# 6. 可重复执行命令

.\.tools\paper-env\Scripts\python.exe .\analysis\rocket_recovery\yang_2026.py report
.\.tools\paper-env\Scripts\python.exe .\analysis\rocket_recovery\yang_2026_landing_identification.py report
.\.tools\paper-env\Scripts\python.exe .\analysis\rocket_recovery\barge_wave_sensitivity.py report
.\.tools\paper-env\Scripts\python.exe .\analysis\rocket_recovery\yang_2026_figures.py
.\.tools\paper-env\Scripts\python.exe .\analysis\rocket_recovery\paper_integrity_audit.py report

HAMS 与 Chrono 的完整重算分别由项目现有 PowerShell 流水线调用本地编译求解器和 .tools/chrono-env。论文制图环境与 Chrono 环境分离，SciencePlots 的安装不修改系统 Python 或系统 PATH。

# 7. 审查结论

现有 HAMS 驳船、JONSWAP 筛选、Yang 公开数据辨识模型和 Chrono-Cummins 代理结果均有机器可读输出和生成代码，可以证明主文中的‘本文计算’并非从参考论文复制。文献值只在验证曲线和输入参数中出现，并已明确标注。与此同时，当前模型仍不具备真实支腿 CAD、经试验标定的缓冲器曲线、可接受的全尺度接触贯入和全 6DOF 强耦合闭合，因此论文必须保持为开放中等保真度框架与分层验证研究，不能写成某型火箭海上回收全过程已完成工程验证。